# Limelight 3A reference

# Limelight 3A reference

Researched on 2026-09-12. These are locally readable reference notes, not a full mirror of the vendor website. Java behavior is checked against the published FTC Hardware 11.2.1 sources, matching this repository's build.dependencies.gradle. Camera firmware was not available for verification; the vendor's online firmware documentation is unversioned.

- Java objects and runtime behavior — start here for the Java library

- API reference and Java example

- Java objects, JSON keys, units, and compatibility notes

## Camera setup

The 3A connects to the Control Hub's blue USB 3.0 port using USB-C to USB-A. It exposes an Ethernet-over-USB interface. Scan for it in Driver Station robot configuration and name it limelight to match the example. Configure pipelines through the browser UI at http://limelight.local:5801 when connected to your computer, or use Limelight Hardware Manager to discover the device. Pipeline slots are 0–9.

This model has no built-in illumination or RJ45 connector. Neural pipelines run on its CPU; Google Coral is unsupported. MegaTag2 uses externally supplied robot orientation. See the official 3A quick-start.

## Sources and refresh

| Source | Purpose |
| --- | --- |
| 3A quick-start | Model-specific wiring and setup |
| FTC programming guide | Pipelines, targeting, and localization |
| 11.2.1 Java API | Class and method reference |
| 11.2.1 source archive | Exact signatures, JSON parsing, defaults, and driver behavior |
| HTTP API | Direct camera requests |
| JSON results | Firmware wire format |
| JSON status | Diagnostic wire format |

When upgrading the SDK, download its matching source archive and compare com/qualcomm/hardware/limelightvision/{Limelight3A,LLResult,LLResultTypes,LLStatus,LLFieldMap}.java. Recheck the vendor pages against the installed camera firmware. In particular, retest pipeline switching, missing results, array lengths, and units with camera output.

## Repository examples

- SDK Limelight sample

- Team tag lookup helpers

- AprilTags and Limelight lesson

ConceptAprilTagLocalization uses the FTC VisionPortal/AprilTagProcessor path. Its detection objects are different from Limelight's LLResultTypes.FiducialResult.