# Limelight 3A API (FTC SDK 11.2.1)

# Limelight 3A API (FTC SDK 11.2.1)

For class responsibilities, result ownership, lifecycle details, field-map construction, and calibration, see Java objects and runtime behavior.

All camera classes below use com.qualcomm.hardware.limelightvision. Signatures and implementation notes were checked against the published 11.2.1 source archive. See also the Limelight3A Javadoc.

## Driver methods

| Signature | Behavior |
| --- | --- |
| void setPollRateHz(int rateHz) | Call before start(); clamps to 1–250 Hz; ignored while running. Default 100 Hz. Polling frequency is not camera frame rate. |
| void start() | Start/resume background result polling. |
| void pause() | Suspend polling. |
| void stop() | Stop polling and shut down its executor. |
| boolean isRunning() | Polling state, not evidence of target detection. |
| boolean isConnected() | Last successful update was at most 250 ms ago. |
| long getTimeSinceLastUpdate() | Milliseconds since last successful update. |
| LLResult getLatestResult() | Cached parsed result; does not request a fresh frame. |
| LLStatus getStatus() | Fetch diagnostics; returns a default object if request fails. |
| boolean pipelineSwitch(int index) | Request pipeline 0–9; inspect subsequent result index before using it. |
| boolean reloadPipeline() | Reload the active pipeline. |
| boolean updateRobotOrientation(double yaw) | Send field-aligned robot yaw in degrees for MegaTag2. |
| boolean updatePythonInputs(double[] inputs) | Accepts 1–32 numbers; rejects null/empty/oversize arrays. |
| boolean updatePythonInputs(double input1, double input2, double input3, double input4, double input5, double input6, double input7, double input8) | Eight-value convenience overload. |
| boolean captureSnapshot(String snapname) | Save a camera image. |
| boolean deleteSnapshot(String snapname) | Delete one snapshot. |
| boolean deleteSnapshots() | Delete all snapshots. |
| boolean uploadPipeline(String jsonString, Integer index) | Upload pipeline JSON. |
| boolean uploadFieldmap(LLFieldMap fieldmap, Integer index) | Upload a field map; rejects a map failing isValid(). |
| boolean uploadPython(String pythonString, Integer index) | Upload script text. |
| LLResultTypes.CalibrationResult getCalDefault() | Default calibration. |
| LLResultTypes.CalibrationResult getCalFile() | Calibration file. |
| LLResultTypes.CalibrationResult getCalEEPROM() | EEPROM calibration. |
| LLResultTypes.CalibrationResult getCalLatest() | Latest calibration. |

Upload methods accept null for the default slot. Boolean command returns report request success; they do not prove that a subsequent image has been processed with the new settings. Commands perform HTTP requests and can block; avoid uploads or snapshot management in a timing-sensitive driving loop.

## Read a specific AprilTag

Original example: copy into a TeamCode Java file named LimelightReadTag.java, configure an AprilTag pipeline in slot 0, and replace the illustrative tag ID. The example only reports telemetry.

package org.firstinspires.ftc.teamcode;

import com.qualcomm.hardware.limelightvision.LLResult;
import com.qualcomm.hardware.limelightvision.LLResultTypes;
import com.qualcomm.hardware.limelightvision.Limelight3A;
import com.qualcomm.robotcore.eventloop.opmode.LinearOpMode;
import com.qualcomm.robotcore.eventloop.opmode.TeleOp;

@TeleOp(name = "Limelight Read Tag")
public class LimelightReadTag extends LinearOpMode {
    @Override
    public void runOpMode() throws InterruptedException {
        final int pipeline = 0;
        final int targetId = 1; // Replace with the desired field tag.
        final long maxAgeMs = 100; // Example policy; tune for your application.
        Limelight3A camera = hardwareMap.get(Limelight3A.class, "limelight");
        camera.setPollRateHz(100);
        if (!camera.pipelineSwitch(pipeline)) {
            telemetry.addLine("Pipeline switch failed");
            telemetry.update();
            return;
        }
        camera.start();
        try {
            waitForStart();
            while (opModeIsActive()) {
                LLResult result = camera.getLatestResult();
                boolean found = false;
                if (camera.isConnected() && result != null && result.isValid()
                        && result.getStaleness() <= maxAgeMs
                        && result.getPipelineIndex() == pipeline) {
                    for (LLResultTypes.FiducialResult tag : result.getFiducialResults()) {
                        if (tag.getFiducialId() == targetId) {
                            telemetry.addData("tx degrees", tag.getTargetXDegrees());
                            telemetry.addData("ty degrees", tag.getTargetYDegrees());
                            found = true;
                            break;
                        }
                    }
                }
                telemetry.addData("Requested tag visible", found);
                telemetry.update();
                idle();
            }
        } finally {
            camera.stop();
        }
    }
}

In 11.2.1, getLatestResult() synthesizes an empty invalid result when its cache is null. Older samples describe null before startup. Keep the null guard, but never use non-null alone to establish validity. A failed poll can leave an old valid result cached. getStaleness() measures time since SDK parsing, not image exposure; repeated responses containing the same camera frame can look newly received. Compare getTimestamp() across iterations when a control algorithm must process each camera frame only once.

MegaTag2 additionally requires ongoing updateRobotOrientation(yawDegrees) calls using an IMU heading aligned to the loaded field map, followed by result.getBotpose_MT2(). Configure camera mounting pose and the correct field map before interpreting either MegaTag pose. See the FTC programming guide.

## HTTP interface

The camera serves HTTP at http://<camera-ip>:5807. Use the camera's IP, not the Control Hub interface address displayed as a serial number. These routes are documented in the official HTTP reference and used by the SDK source.

| Method and route | Request / response |
| --- | --- |
| GET /results | Current targeting JSON; see data shapes. |
| GET /status | Diagnostic JSON. |
| GET /hwreport | Hardware/calibration reports. |
| POST /pipeline-switch?index=0 | No body; select pipeline. |
| POST /update-pythoninputs | JSON numeric array, such as [1, 0, 0, 0, 0, 0, 0, 0]. |
| POST /update-robotorientation | SDK sends [yaw, 0, 0, 0, 0, 0]. |

From a computer that can reach the camera, a read-only diagnostic request is:

curl --max-time 2 http://limelight.local:5807/results
curl --max-time 2 http://limelight.local:5807/status