# REV DUO Power Transmission & Motion Guide

# REV DUO Power Transmission & Motion Guide

Winsor Robotics FTC Resource Directory
Component: Gears, Sprockets & Chain, Timing Belts, and Wheel Systems

Power transmission components move mechanical energy from motors to where it is needed on the robot. Because REV DUO is built on a 15mm structural grid, its gears, sprockets, and pulleys are mathematically designed to mesh at integer multiples of 15mm.

## 1. 0.5 Module Gears & The 15mm Grid

REV DUO spur gears use the Metric Module 0.5 (0.5 Mod) tooth profile with a standard 5mm hex bore.

### Gear Mathematics

- Pitch Diameter (PD):

PD=Teeth×Module=N×0.5 mm=N2 mm

- Center-to-Center Distance (CD):

CD=PD1+PD22=N1+N24 mm

### How REV Gears Align with 15mm Extrusion & Channels

Notice the symmetry: any gear pair whose tooth counts sum to a multiple of 60 meshes at a multiple of 15mm!

| Driver Gear | Driven Gear | Sum of Teeth | Center Distance (CD) | Extrusion / Channel Alignment |
| --- | --- | --- | --- | --- |
| 20T | 40T | 60 | 15.0 mm | Exactly 1 standard 15mm extrusion width! |
| 40T | 80T | 120 | 30.0 mm | Exactly 2 standard extrusion widths (30mm)! |
| 20T | 100T | 120 | 30.0 mm | Exactly 2 standard extrusion widths (5:1 reduction)! |
| 60T | 60T | 120 | 30.0 mm | 1:1 idler spacing |
| 30T | 90T | 120 | 30.0 mm | 3:1 reduction |

### Gear Assembly Best Practices

- Never run gears dry without grease: Apply a light coat of white lithium grease or teflon lube to reduce friction and eliminate gear whine.

- Prevent gear climbing: Under high torque, plastic gear teeth can deflect and skip over each other. For high-torque joints (such as arm pivots), clamp a High Strength Hex Hub (REV-41-1147) onto the gear face using M3 screws and nyloc nuts.

- Idler Gears: To bridge a long distance between shafts, insert an idler gear. Idler gears change the direction of rotation but do not alter the gear ratio.

## 2. Sprockets and #25 Roller Chain

Chain drives are the gold standard for transferring torque across long spans where gears are too bulky or heavy (such as drivetrains and lift winches).

### Chain Specifications

- Chain Size: Standard ANSI #25 Roller Chain (0.25" / 6.35mm pitch).

- Sprocket Sizes: 10T, 15T, 20T, 30T, and 40T with 5mm hex bore.

- Materials: Acetal (lightweight, intake/roller use) and Sintered Steel/Aluminum (drivetrains and high-shock loads).

### REV Chain Tool (REV-41-1442)

The REV chain tool allows builders to push pins out of #25 chain and press them back together without needing weak master links.

- Breaking Chain: Place the chain link in the slot, align the drive pin with the chain pin, and turn the handle until the pin pushes out.

- Assembling Chain (Master-less): Leave the pin held partially in the outer plate! Slip the new link into place and use the reverse press on the tool to push the pin back flush.

### Chain Tensioning Rules

- Chain stretches over time: Brand-new #25 chain will elongate slightly during the first few matches ("run-in").

- Always incorporate tension adjustment: Never fix both shaft bearings at completely stationary points without a mechanism to tension the chain. Use:

  - Sliding bearing brackets along 15mm extrusion slots.

  - Half-links (REV-41-1366) to achieve precise link counts.

  - Floating Delrin chain idler blocks or floating sprockets.

- Target Sag: A properly tensioned FTC chain run should have approximately ¼ inch (6mm) of vertical flex under moderate finger pressure. Overtightened chain causes excessive motor friction; loose chain will jump teeth and derail.

## 3. GT2 Timing Belts & Pulleys

Timing belts offer zero backlash, require zero lubrication, and run near-silently at high RPM.

### Specifications

- Belt Standard: GT2 (3mm pitch), 9mm width neoprene with fiberglass reinforcement.

- Pulleys: 15T, 24T, 30T, and 36T with 5mm hex bore and side retention flanges.

### When to Choose Belts over Chain:

- Flywheel Shooters: At 2,000–6,000 RPM, roller chain will oscillate violently and throw lubricant. Belts run smoothly and stably at high speeds.

- High-Speed Intakes: Belts absorb impact when colliding with game elements without snapping.

- Precision Autonomous Encoders: Belts have virtually zero mechanical backlash compared to chain slop.

## 4. Wheel Selection & Ground Interaction

REV DUO offers four wheel families: Traction/Grip, Omni-Directional, Mecanum, and Compliant.

[Traction]             [Omni]                 [Mecanum]             [Compliant]
 (Solid Tread)      (Perpendicular Rollers)  (45° Angled Rollers)    (Flexible Urethane)
  High Grip            Zero Scrub Turns       Holonomic / Strafe      Intake Compression

### Wheel Comparison Matrix

| Wheel Type | Available Sizes | Main Benefit | Main Trade-off | Ideal Use Case |
| --- | --- | --- | --- | --- |
| Traction / Grip (REV-41-1354) | 75mm, 90mm | Maximum pushing force, zero slip | Cannot slide sideways | Center-drop tank drive, defense bots |
| Omni-Directional (REV-41-1190) | 75mm, 90mm | Eliminates turning scrub | No lateral resistance (pushed easily) | Outer wheels on tank drive, odometry pods |
| Mecanum (REV-45-1655) | 75mm, 90mm | Full 360° translation (strafing) | Reduced forward tractive force | General FTC competition holonomic drive |
| Compliant | 35mm, 50mm, 75mm, 100mm | Deforms around game objects | Not suitable for driving on the floor | Roller intakes, ball collectors, ramps |

### The Mecanum Wheel "O-Pattern" Rule

Important

MECANUM ROLLER ORIENTATION:
Mecanum wheels come in distinct Left and Right configurations based on the 45-degree angle of the perimeter rollers.

When viewed from above looking down on the robot:

- The contact rollers touching the carpet must form an "X" shape.

- The top-facing rollers must form an "O" or diamond pointing outwards toward the corners.

Front Left  [/]     [\]  Front Right
               \       /
                \     /
                 \   /
                  \ /
                   X
                  / \
                 /   \
                /     \
               /       \
  Back Left   [\]     [/]  Back Right

If two wheels on the same side are swapped, the robot will drive diagonally when commanded forward or spin uncontrollably when trying to strafe!

### Compliant Wheel Durometer Guide

Compliant wheels are made of flexible thermoplastic polyurethane (TPU) and deform around game pieces to create high friction:

- 30A (Green — Soft): Extremely squishy. Best for hard, slick plastic spheres or irregularly shaped game elements. Conforms immediately to game piece geometry.

- 45A (Grey/Black — Medium): General-purpose intake roller. Good balance of grip and mechanical durability.

- 60A (Orange — Firm): Stiff. Best for high-wear feeder wheels and heavy game pieces where the wheel must maintain its circular geometry under speed.

## Official Links & References

- Introduction to Motion (Docs)

- Gears and Gear Math

- Advanced Gears & Addendum Shifting

- Sprockets and Chain Guide

- Chain Tool Instructions

- Timing Belts and Pulleys

- REV Wheel System Overview

- Mecanum Wheel Behavior & Setup Guide